# Lab Protocol: Working with Media

- Before using media:

  - Always label media before inoculation. Use a Sharpie for plates and labeling tape for tubes.

  - Label the bottom of the plate (the part of the plate that holds the agar).

  - Label the perimeter of one agar plate on the bottom or agar side (not the cover) or labeling tape for tubes.  Include:

    - First and last name; do not use initials.

    - Date of inoculation

    - Type of medium (may already be present)

    - Incubation temperature in Celsius

    - Brief description of inoculum or sampled environment.

Figure 1. A properly labeled agar plate.

- After inoculation:

  - Unless specifically instructed otherwise, place plates inverted (upside down) for incubation.

  - This prevents condensation from falling on the surface of the agar and disrupting the streaking pattern or encouraging contamination.

- Inoculation techniques:

  - An Exposure technique is a simple way to collect environmental microbes that involves exposing an agar plate with growth medium to a specific environment for a particular timeframe.  This technique is commonly utilized in one of two ways:

- Demonstrating bacterial growth over time – for example, you could expose five agar plates to a bathroom environment at the same time.  The first plate would be capped at 5 minutes, the second at 10 minutes and so on.  The hypothesis might state that you’d expect more bacterial growth on the plates that were exposed for the longest timeframe.

- Demonstrating bacterial growth over distance – for example, you could expose five agar plates within a five-foot radius of a toilet.  Each plate would be placed within a foot of each other but with a specific and uniform exposure time for each plate.  The hypothesis might state that you’d expect more bacterial growth on the plates located closer to the toilet than further away.

- Pros/Cons:  This is simple to use and easy to control most experimental variables.  Unfortunately, this technique is prone to growth of undesirable contaminants that may negatively impact experimental results.

  - A Direct contact technique involves applying an object of interest directly on the surface of an agar plate or immersed into a tube.

  - A Swab technique involves swabbing objects using either a dry or moist collection method.  The swab is used to inoculate the surface of an agar plate or broth tube.

- Moist technique involves dipping a swab in a tube of sterile water prior to collection.

- Dry technique uses a swab without adding moisture.

- Pros/Cons:  A moist collection will generally increase microbial acquisition.  It introduces a potential contamination issue, however.  It may also be less than ideal depending on the material or area swabbed or impractical for a given location.

- Inoculation Do’s and Don’ts:

  - Avoid talking while the agar is exposed. Bacteria are expelled from your mouth and can contaminate the growth surface.

  - Lids are held in the non-dominant hand and never placed on the bench.

  - Use the lid like a shield to prevent airborne bacteria from touching your plate. The agar side of the plate should not be waived around in the air.

  - Dispose of applicator swab in the regular trash immediately. Used swabs cannot be placed on bench after use. This contaminates the work surface.