# Name:

# Lab Exercise: Gram Stain

Bacteria have almost the same refractive index as water. This means that when you try to view them using a microscope they appear as faint, gray shapes and are difficult to see.

Staining cells makes them easier to visualize.

# Steps to Gram Staining:

- Prepare a smear.

- Gather stain supplies including a beaker, stains, blotting paper, and microscope.

- Steps of the Gram Stain:

Step 1 – Flood prepared smear with Crystal Violet. Wait 60 seconds. Step 2 - Pour off excess stain and briefly rinse with water.

Step 3 – Flood smear with Gram’s Iodine. Wait 60 seconds. Step 4 - Pour off excess iodine and briefly rinse with water.

Step 5 - Drop Ethanol onto angled slide until it runs clear. (Do not over do it!) Step 6 – Flood smear with Safranin. Wait 60 seconds.

Step 7 - Pour off excess stain and briefly rinse with water.

- Stained slides do not need to be air dry to visualize under a microscope. Tap the slide’s side against blotting paper to wick off excess moisture.

- View slide under 4x, 10x and 40 x objectives. When confident that you are visualizing a microbe, prepare to use the oil immersion objective:

  - Turn the nose piece between the 40x and 100x objective, place 1-2 drops of oil directly on the slide.

  - Click the 100x objective into place.

    - The oil will touch the objective as it occupies the space between the slide and the objective.

    - Utilize fine focus as needed to view under the 100x objective.

- Gram positive bacteria retain a purple color. Gram negative bacteria will look red or pink.

- Bacterial slides are placed in the Sharps container.

- 100x microscope objectives must be ‘pumped’ to remove immersion oil. Be sure to only use optical wipes to prevent scratching the objectives  Inspect and clean the scope to remove any excess stain or oil from the stage or knobs.